Firmware Upgrade Guide for Oscilloscope Series

- This guide applies to the following models: MSO1000HD series, MSO3000HD series, MSO5000HD series, UPO1000HD series, UPO2000HD series, MSO3000X, and MSO2000X series.

- The software version of the upgrade file must be greater than or equal to the current software version of the oscilloscope.

Warning: Do not power off the oscilloscope during the upgrade process.

Upgrade Steps

- Copy the corresponding upgrade file (e.g., mso3000hd_all_1.01.00XX_armv8a.ipk) that matches the oscilloscope model to a USB flash drive.

- Insert the USB flash drive into the USB port of the oscilloscope.

Upgrade Methods

Method 1

- Restart the oscilloscope and enter the upgrade interface.

- Use the multi-function rotary knob to select the upgrade file.

- Press the multi-function knob to start the upgrade process, as shown in the following figure.

- Once the upgrade is complete, remove the USB flash drive and restart the oscilloscope.

Method 2

- Navigate to the upgrade option in the Auxiliary interface.

- Select the upgrade file that matches the oscilloscope model.

- Click the Upgrade button to start the process.

- Wait for the upgrade to be completed. If successful, the oscilloscope will automatically restart.

- Once finished, remove the USB flash drive.

Verification

- If the upgrade fails, check whether

- The firmware file corresponds to the correct oscilloscope model.

- The firmware version is compatible with the current software version.

- After restarting, navigate to Auxiliary → About this oscilloscope to check the software version number to confirm whether the upgrade was successful.