UDP3000S DC power supply firmware upgrade guide

- Select the upgrade file according to the model and put it in the root directory of the FAT32 format U disk (the file name cannot be modified).

- Insert the USB flash drive into the machine to be upgraded then power off.

Note: Use USB port at back of machine. Not front USB port.

- Press and hold the "UTILITY" button and hit power button to turn on the power supply and start the upgrade.

- After the upgrade is completed, Remove USB and manually restart the machine, by hitting power button.

Language change procedure.

- If after upgrade the unit defaults to Chinese. Follow this procedure.

- Press “Utility” button

- Use knob and rotate to Setup. (4th menu down)

- Use knob and rotate to desired language: Chinese, English, Deutsch